# Install Ubuntu Server 24.04

- Download Raspberry Pi Imager

- Select Raspberry Pi 5

- Select Other General Purpose OS

- Select Ubuntu

- Select Ubuntu Server 24.04.4 LTS

- Set username and password to: robotics

- Set WiFi to a phone or computer’s personal hotspot

- Enable SSH using password authentication

- Write to microSD card

## Setting up Network Manager (I only know how on Windows)

This step is optional, using netplan to connect to Wi-Fi is not as elegant as Network Manager, but it works.

- Connect Pi to computer using ethernet and see happy blinking ethernet lights

- Go to View Network Connections

- Open the properties for your computer’s internet connection

- Share internet connection with Pi

  - In my case, the Pi’s ethernet was connected to Ethernet 2

  - Note the IP address

- Find the Pi’s IP address

  - Open terminal and run: arp -a

  - Look for the interface with the IP address from earlier

  - Try pinging the devices that start with 192.168.137.XXX

    - ping 192.168.137.36

  - Identify the IP addresses that sends a reply

    - In my case it was 192.168.137.36

- Connect to Pi through ssh

  - ssh robotics@192.168.137.36

  - Password should be: robotics

- Hopefully we’re in!

- Install NetworkManager

  - sudo apt update

  - sudo apt upgrade

  - sudo apt install network-manager

- Switch network renderer from netplan (default) to NetworkManager

  - cd /etc/netplan

  - sudo nano 50-cloud-init.yaml or any file that’s there

  - Change renderer: NetworkManager

  - Save and exit

  - sudo netplan apply

  - sudo reboot now

  - Hopefully you can use sudo nmtui to change network settings!

    - If not, hopefully the ethernet still works so you can connect to the Pi.

  - These are my NetworkManager settings for the ethernet port

- Set Static IP addresses

  - Set a static IP address using nmtui to: 192.168.0.42

  - Set static IP address on your computer’s ethernet connection

    - IP address: 192.168.0.xxx

      - Xxx can probably be any number between 20 – 255

      - Don’t set it to 192.168.0.42 because that’s the Pi’s address

    - Subnet mask: 255.255.255.0

  - Now, ssh robotics@192.168.0.42, should work over ethernet!

## Set up serial port

- cd /boot/firmware

- Edit config.txt

  - Add dtoverlay=uart0,ctsrts underneath enable_uart=1

  - Save file

- Check cmdline.txt

  - Check if it contains console=serial0,115200 and console=tty1

- Reboot pi

  - sudo reboot now

- Now /dev/ttyAMA0 should hopefully point to GPIO pins 14 and 15